# BESS Raw Materials and India Status

Status as of 23 October 2026

# Summary

A BESS is mostly an LFP battery plus the equipment around it. Its raw materials come in two layers: what goes into the cells, and what goes into the container system around them.

# 1. Cell Materials (LFP Chemistry)

| Component | Raw material | Rough amount per kWh* | India status |
| --- | --- | --- | --- |
| Cathode | LFP: lithium carbonate + iron phosphate (iron ore, phosphate rock) | ~2 kg LFP; ~0.5-0.6 kg lithium carbonate | Lithium fully imported; LFP cathode starting (Himadri, GFCL, Altmin, Epsilon) |
| Anode | Graphite (synthetic or natural) | ~1-1.2 kg | Small: Epsilon 2,000 t, Himadri 200 t; rest imported |
| Electrolyte | LiPF6 salt + carbonate solvents | ~0.6-0.8 kg | Only part bought locally (GFCL, Neogen; solvents from Alkyl Amines, Balaji Amines) |
| Separator | Polyethylene/polypropylene film, ceramic-coated | Thin film | Imported |
| Current collectors | Aluminium foil (cathode), copper foil (anode) | ~0.25 kg Al; ~0.5 kg Cu | Al foil: Hindalco (supports ~100 GWh); Cu foil imported |
| Binder / additive | PVDF, conductive carbon | Small | PVDF: GFCL nearing qualification |
| Cell can | Aluminium casing | - | Available |

*Typical industry ranges, not from a single cited source.

Cathode and anode together are about 65% of cell cost, and more than 90% of these materials are processed in China.

# 2. System Materials (Container Around the Cells)

Modules and racks: steel and aluminium structures, busbars (copper or aluminium), cabling.

Power conversion system (PCS) / inverter: IGBT power semiconductors, copper magnetics, capacitors. Mostly imported; some assembled in India.

BMS and EMS: electronics and software. EMS counts towards the 20% domestic-content rule for VGF.

Thermal management: liquid-cooling plates, coolant, chillers.

Fire safety: detection and suppression systems.

Container: steel ISO-style enclosure; can be made in India.

Grid side: transformers and switchgear; strong local base in India.

# 3. Why It Matters for India

Simpler than EV NMC: BESS uses LFP, which needs no nickel or cobalt. India's critical-mineral gap is mainly lithium and battery-grade graphite.

Local value today: India can already supply the container, transformers, structures, aluminium foil and EMS. The high-value parts (cell, cathode, anode, separator) are still imported.

Sodium-ion as an alternative: replaces lithium with sodium and graphite with hard carbon. Epsilon has a coconut-shell hard carbon; Reliance owns Faradion.

# Sources

1. BusinessToday - India has failed to build an ecosystem of battery cells; component PLI (22 Oct 2026) - https://www.businesstoday.in/auto/story/bt-big-story-india-has-failed-to-build-an-ecosystem-of-battery-cells-can-a-new-pli-change-this-562898-2026-10-22

2. ORF America - India's potential role in global battery supply chains - https://orfamerica.org/newresearch/batteries-ev-india

3. Indian Chemical News - Hindalco commissions battery foil facility - https://www.indianchemicalnews.com/battery/lite/hindalco-to-invest-rs-21000-crore-on-smelter-expansion-commissions-frp-and-battery-foil-facilities-in-odisha-29082

4. GFCL investor presentation, May 2026 - https://nsearchives.nseindia.com/corporate/FLUOROCHEM_26052026164956_GFCL_Investor_Presentation_26May2026.pdf

5. Indian Chemical News - Epsilon Group interview (anode, LFP cathode, hard carbon) - https://www.indianchemicalnews.com/interviews/india-must-act-now-to-dominate-global-battery-materials-vikram-handa-managing-director-epsilon-group-581

6. Energetica India - Alternative battery supply chain (electrolyte solvents) - https://www.energetica-india.net/articles/indias-growing-dominance-in-the-ev-sector-a-global-shift-towards-an-alternative-supply-chain

7. Energy-Storage.news - 20% domestic content rule for BESS VGF - https://www.energy-storage.news/?p=92648